XYLOSOME

UI Concept & Wishlist

Temporal scanning unit  ·  pendant HMI  ·  Qt6 / QML

| Document type | Concept / working draft |
| --- | --- |
| Version | 0.1 · 2026-05-20 |
| Display | WaveShare 5.5" · 960 × 540 px · HDMI + USB touch |
| Pi application | Qt6 / QML · pi/hmi/ · 115200 USB serial ← Teensy |
| Motion control | ClearCore → Panasonic servo + NEMA 17 stepper |
| Imaging | Line scanner camera · trigger sync via ClearCore |
| Pendant controls | Teensy 4.1 carrier PCB Rev B · Grayhill encoder · 3 buttons |

This document defines the intended screens, controls, data displays, and wishlist items for the Xylosome HMI. It is a living reference — update it as the design evolves.

Design direction: touch-free target The display will ultimately be used as output only — no touch input. Every interaction must be achievable via encoder rotate, encoder click, BTN1, and BTN2. Touch support remains in the current codebase as a development convenience but should not be relied upon for any primary workflow. See Section 4.5 for the full gesture mapping.

# 1.  System Overview

The Xylosome is a handheld scanning unit. The pendant contains a Raspberry Pi 5 running a Qt6/QML touch interface on a 5.5" WaveShare display. A Teensy 4.1 reads the physical controls (jog wheel + 3 buttons) and pipes events to the Pi over USB serial. The Pi talks to a ClearCore motion controller in the main box over TCP/Ethernet via a single CAT6 PoE cable up to 100 m.

## 1.1  Components under UI control

| Panasonic servo | Primary scanning axis — position, velocity, torque control |
| --- | --- |
| NEMA 17 stepper | Secondary axis — role TBD (focus / Z / indexing) |
| Line scanner camera | Trigger timing, exposure, line rate — coordinated by ClearCore |
| ClearCore | Motion sequencer — receives high-level commands from Pi via TCP |
| Pendant controls | Jog wheel, BTN1, BTN2, ENC_SW — context-sensitive per active screen |

## 1.2  Screen map

| ScreenSplash | Boot animation / connection check — auto-advances to ScreenScan |
| --- | --- |
| ScreenScan | PRIMARY — velocity profile editor + execute scan (was ScreenSequences) |
| ScreenLive | Manual jog and real-time telemetry for both axes |
| ScreenCamera | Line scanner settings — exposure, line rate, trigger mode (NEW) |
| ScreenPresets | Saved axis positions and scan configurations (stub exists) |
| ScreenTelemetry | Live graphs — position, velocity, current, temp (stub exists) |
| ScreenSettings | System config — network, calibration, axis params, pendant mapping |
| ScreenHome | Navigation menu — accessed via [settings] from any screen |

# 2.  Screens

| ScreenSplash boot screen · auto-advance to ScreenScan | EXISTS |
| --- | --- |

### Purpose

Shown on startup. Checks connections to ClearCore and Teensy, then advances automatically to ScreenScan.

### Displays

XYLOSOME wordmark / logo

Connection status indicators: ClearCore TCP, Teensy USB, camera

Progress spinner while connecting

### Wishlist

⚠  Show live connection status — green tick when ClearCore TCP handshake succeeds

⚠  If ClearCore unreachable after timeout, stay on splash with a Retry button

⚠  Display ClearCore firmware version once connected

| ScreenScan primary screen · velocity profile editor · was ScreenSequences | EXISTS |
| --- | --- |

### Purpose

The main working screen. The operator programs a velocity profile for the scan axis using a Catmull-Rom spline editor, then executes the scan. The spline Y-axis represents scan speed; X-axis is time / travel. The line scanner camera is triggered by ClearCore in sync with axis motion.

### Displays

Full-width spline canvas (top half of screen)

Axis position dial — start hand / end hand / playhead tracking hand

Live line count, aspect ratio, average velocity overlay on canvas

Execution state badge: idle / running / paused

### Controls

Drag spline nodes — [touch] drag  |  [encoder] focus section → select node → ENC rotate to reposition

Drag handle bar (boxW) — [touch] drag  |  [encoder] focus Position section → boxW parameter → ENC rotate

Insert node — [touch] double-tap canvas  |  [encoder] scrub bar → ENC click at desired position

Delete node — [touch] double-tap node  |  [encoder] focus node → BTN1 (endpoints immune)

[execute] — start scan sequence

[pause] / [resume] — pause mid-scan and resume

[home] — move axes to home position before scan

[settings] — navigate to ScreenHome menu

### Pendant behaviour

| Input | Action | Notes |
| --- | --- | --- |
| JOG | Scrub playhead (preview) or adjust focused value | Context depends on active sub-level (see Sec. 4.3) |
| BTN1 | Toggle execute / pause OR delete focused node | Action depends on sub-level — execute at top, delete in node-focus |
| BTN2 | Back one level (top level: home axes) | Consistent back at any depth; home only when at section-cycle level |
| ENC_SW | Enter / confirm focused item | Same as ENC click |

### Wishlist

⚠  Wire [execute] to real ClearCore TCP commands — replace mock playhead with actual axis feedback

⚠  Live camera line counter during active scan

⚠  Scan duration estimate calculated from velocity profile and boxW

⚠  Save / load named scan profiles (persistent JSON on Pi)

⚠  Visual trigger-point markers along the timeline

⚠  Axis soft-limit zones shown as red shading on canvas edges

| ScreenLive manual axis control · real-time telemetry | EXISTS — mock data |
| --- | --- |

### Purpose

Manual jogging and real-time monitoring of both axes. Used for setup, homing, testing, and fault investigation.

### Displays

Active axis selector (Panasonic servo / NEMA 17 stepper)

Mode selector: position / velocity / torque

Setpoint value and slider (−100 to +100, maps to axis range)

Telemetry: position, velocity, current, temperature per axis

Enable / disable status indicator

### Controls

Axis selector toggle

Mode dropdown — position, velocity, torque

Setpoint slider

[enable] / [disable]

[zero] — zero the setpoint

### Pendant behaviour

| Input | Action | Notes |
| --- | --- | --- |
| JOG | Jog active axis | Rate TBD — fine/coarse modes |
| BTN1 | Cycle active axis (servo / stepper) | Visual feedback updates on screen |
| BTN2 | Toggle enable / disable |  |
| ENC_SW | Zero setpoint |  |

### Wishlist

⚠  Split view — show both axes simultaneously rather than toggling

⚠  Jog speed modes — fine / coarse (hold BTN1 while jogging for fast)

⚠  Soft limit indicators — visual warning when approaching axis limits

⚠  Replace mock tick() with real telemetry from ClearCore over TCP

⚠  Axis fault indicator — highlight in red if ClearCore reports a fault

| ScreenCamera line scanner settings · new screen — not yet built | TODO |
| --- | --- |

### Purpose

Configure the line scanner camera. All camera timing and triggering is coordinated by the ClearCore, so settings here translate to ClearCore configuration commands sent over TCP.

### Displays

Camera connection status and model

Current exposure time

Line rate (lines per second)

Trigger mode: free-running / ClearCore encoder-locked

Image buffer status

Live single-line grab preview (updates ~10 Hz for focus/exposure check)

### Controls

Exposure slider / numeric entry

Line rate input — set lines/sec or link to axis velocity

Trigger mode toggle

Gain control

[grab test line] — capture one line for focus/exposure verification

### Pendant behaviour

| Input | Action | Notes |
| --- | --- | --- |
| JOG | Adjust focused parameter (exposure or line rate) | Active parameter selected by ENC_SW |
| BTN1 | Grab test line |  |
| BTN2 | Toggle trigger mode |  |
| ENC_SW | Cycle active parameter |  |

### Wishlist

⚠  Auto-calculate line rate from the ScreenScan velocity profile — link the two screens

⚠  Exposure histogram overlay on the test line grab

⚠  Auto-exposure mode — ClearCore / camera feedback loop

⚠  Camera model and serial number shown in screen header once connected

| ScreenPresets saved positions and scan configurations | STUB |
| --- | --- |

### Purpose

Quick access to saved axis positions and complete scan configurations. Speeds up repeated setups.

### Displays

List of named presets with type (position / full scan config)

Selected preset detail view — axes, velocity profile, camera settings

### Controls

[go to] — move axes to preset positions

[load scan] — populate ScreenScan with saved velocity profile

[save current] — save current state as a new named preset

[delete]

### Wishlist

⚠  Persistent storage — presets saved to JSON file on Pi, survive reboots

⚠  Jog-wheel quick recall — spin encoder to select, ENC_SW to confirm

⚠  Import / export preset files over network or USB drive

| ScreenTelemetry real-time monitoring and history | STUB |
| --- | --- |

### Purpose

Live graphs of motor telemetry for monitoring performance, diagnosing issues, and verifying scan quality.

### Displays

Rolling time-series graphs: position, velocity, current, temperature — per axis

Min / max / average over the last scan cycle

ClearCore TCP connection latency

Fault log — timestamped list of errors reported by ClearCore

### Wishlist

⚠  Graph time window selector — 5 s / 30 s / full scan

⚠  Export telemetry log as CSV over network

⚠  Overlay actual velocity against the programmed velocity profile from ScreenScan

⚠  Temperature warning threshold — configurable in Settings

| ScreenSettings system configuration | EXISTS — needs update |
| --- | --- |

### Purpose

Device configuration. Currently has stale references to the old architecture and needs updating.

### Stale entries to fix

uart.peer = teensy / 1 mbps   →   usb.pendant = teensy / 115200

motor.bus = can / rmd-x8   →   motion = clearcore / tcp

### Configuration items needed

Network — ClearCore IP address or hostname

Network — HTTP server port (currently :8080)

Display — brightness (slider exists; hook to dbus call)

Axes — soft limits for servo and stepper (min/max deg or mm)

Axes — jog speed (fine / coarse step size)

Axes — homing direction and speed per axis

Camera — default exposure, gain, trigger mode

Pendant — button mapping per screen (BTN1 / BTN2 / ENC_SW actions)

Calibration — zero offset for each axis

### Wishlist

⚠  Settings persisted to JSON file on Pi — survive reboots

⚠  Auto-discover ClearCore on LAN via mDNS or fixed IP entry

⚠  System info panel — Pi CPU/memory, uptime, Qt and firmware versions

⚠  Factory reset option

# 3.  Pendant — Global Behaviour

The pendant Teensy sends raw events (JOG delta, BTN DOWN/UP) over USB serial at 115200 baud. The Pi interprets them in the context of the active screen. The table below defines the global behaviour that applies on every screen unless overridden by screen-specific mapping.

| Input | Action | Notes |
| --- | --- | --- |
| JOG (any screen) | Scroll / adjust focused parameter | Context-specific per screen |
| BTN1 long press | Navigate back / return to ScreenScan | Hold 500 ms from any screen |
| BTN2 long press | Soft e-stop command to ClearCore | Hold 1 s — does NOT replace hardwired NC loop |
| ENC_SW (any screen) | Confirm / select highlighted item |  |
| BTN1 + BTN2 | Toggle screen lock (kiosk mode) | Prevents accidental input during live scan |

Note: there is no e-stop button on the pendant PCB. The hardwired e-stop NC loop is external and wired directly into the ClearCore 24V E-STOP IN. The soft command above is a supplementary convenience only.

# 4.  Pendant Navigation Model

The pendant has three physical inputs beyond the jog wheel: ENC_SW (encoder push), BTN1, and BTN2. The interaction model below applies across all screens. It gives the operator a consistent mental model: rotate to browse, push to enter, BTN2 to go back.

## 4.1  Input grammar

| ENC rotate | Move focus — cycle between selectable sections, items, or parameters |
| --- | --- |
| ENC click | Enter / confirm / advance — descend one level into the focused item, or confirm a value |
| BTN1 | Context action — function depends on active screen (see per-screen pendant tables) |
| BTN2 | Back — always returns to the previous focus level or parent screen |

## 4.2  ScreenScan — four-section focus layout

ScreenScan is divided into four focusable sections. The encoder cycles through them; ENC click enters the focused section. BTN2 always exits back to section-cycle mode.

| Section 1 | Spline editor — velocity profile curve |
| --- | --- |
| Section 2 | Position settings — start position, end position, axis parameters |
| Section 3 | BTN1 function target — context action routed to BTN1 for this screen |
| Section 4 | BTN2 function target — secondary context action (overrides Back only at top level) |

## 4.3  Spline editor — sub-navigation flow

Once inside the spline section (ENC click from Section 1), the operator moves through a sequential sub-flow. BTN2 steps back one level at any point.

| Step | Mode | Description |
| --- | --- | --- |
| 1 | Aspect select | ENC rotate cycles through selectable curve aspects (start speed, peak speed, end speed, curve shape, …). The focused aspect is highlighted on screen. |
| 2 | Value edit | ENC click on a highlighted aspect enters edit mode. ENC rotate changes the value; the curve updates live. ENC click again confirms and commits the new value. |
| 3 | Scrub bar | After confirming a value the focus advances to the scrub bar. ENC rotate moves the playhead position left / right along the timeline. The canvas shows the current playhead position and the axis position it corresponds to. |
| 4 | Speed node | ENC click on the scrub bar drops or selects the nearest speed node at the current playhead position. ENC rotate then adjusts the node speed value. ENC click confirms. |
| 5 | Return | ENC click after confirming a node returns focus to the scrub bar. BTN2 at the scrub bar returns to aspect select. BTN2 at aspect select returns to section-cycle mode. |

## 4.4  Navigation state diagram (text)

Indentation shows hierarchy. BTN2 always moves up one level.

ScreenScan — section cycle  [ENC rotate]

└─ Section 1: Spline  [ENC click]

└─ Aspect select  [ENC rotate to cycle]

└─ Value edit  [ENC click → ENC rotate → ENC click to confirm]

└─ Scrub bar  [ENC rotate to position]

└─ Speed node  [ENC click to drop/select → ENC rotate → ENC click to confirm]

└─ Section 2: Position settings  [ENC click]

└─ Parameter cycle  [ENC rotate]

└─ Value edit  [ENC click → ENC rotate → ENC click]

└─ Section 3 / 4: BTN function assignments  [ENC click to reassign]

⚠  This model generalises to other screens — every screen with parameter sub-levels uses the same rotate/click/BTN2 pattern

## 4.5  Touch-free target — gesture mapping

The table below maps every current touch interaction to its encoder equivalent. When an encoder path is marked TBD it must be designed before touch support can be removed. The display (WaveShare 5.5" HDMI) becomes a pure output device; the touch layer can be disabled in Qt or replaced with any non-touch HDMI panel.

| Touch gesture | Screen | Encoder path | Status |
| --- | --- | --- | --- |
| Tap button ([execute], [home], [pause]) | ScreenScan | BTN1 short press (context-mapped) | Defined |
| Drag spline node left/right | ScreenScan | Focus node via section → ENC rotate to reposition | Defined |
| Drag spline node up/down (speed) | ScreenScan | Aspect select → value edit → ENC rotate | Defined |
| Drag handlebar (resize boxW) | ScreenScan | Position section → boxW → ENC rotate | Defined |
| Double-tap canvas → insert node | ScreenScan | Scrub bar → position playhead → ENC click | Defined |
| Double-tap node → delete node | ScreenScan | Focus node → BTN1 | Defined |
| Tap mode selector (pos/vel/torq) | ScreenLive | ENC rotate to cycle modes while mode-row focused | TBD |
| Drag setpoint slider | ScreenLive | Focus slider → ENC rotate | Defined |
| Tap [enable] / [disable] | ScreenLive | BTN2 (mapped globally to toggle enable) | Defined |
| Tap axis selector toggle | ScreenLive | BTN1 (cycle axis) | Defined |
| Tap exposure slider | ScreenCamera | Focus slider → ENC rotate | Defined |
| Tap [grab test line] | ScreenCamera | BTN1 | Defined |
| Tap preset in list | ScreenPresets | ENC rotate to scroll list → ENC click to select | Defined |
| Tap [go to] / [load scan] | ScreenPresets | ENC cycle actions when preset focused → ENC click | TBD |
| Tap settings field / value | ScreenSettings | ENC rotate to field → ENC click → ENC rotate value | Defined |
| Swipe or scroll | Any long list | ENC rotate to scroll | Defined |
| Tap [settings] nav button | Any screen | Long-press BTN1 (500 ms) → ScreenHome | Defined |

⚠  Items marked TBD must be resolved before touch can be fully disabled — primarily list-action selection and mode toggling

⚠  Consider a visual focus cursor (highlight ring or underline) on the active element so the operator always knows what ENC rotate will affect

# 5.  Open Questions

| NEMA 17 role | What does the stepper axis do? Focus / Z / indexing? This determines how it appears in the Live and Scan screens. |
| --- | --- |
| Camera model | Which line scanner? The API and trigger interface define the entire ScreenCamera implementation. |
| ClearCore protocol | What TCP command format does the ClearCore firmware use? This defines the Pi ↔ ClearCore API layer. |
| Scan completion | What happens at end of scan? Auto-return home? Camera stops? The UI needs to reflect this state machine. |
| Jog speed | Fine vs coarse jog — is a single speed sufficient, or should BTN1+JOG give a fast mode? |
| Preset storage | JSON on Pi filesystem only, or sync to ClearCore EEPROM as well? |
| Web UI scope | The HTTP server on :8080 mirrors the touch UI. Should it stay in full sync as the app grows, or become read-only telemetry? |
| Screen lock | Is a kiosk lock mode needed to prevent accidental input during a live scan? |

# 6.  Implementation Status

| Item | Status | Notes |
| --- | --- | --- |
| Qt6/QML HMI framework | Done | Solid structure, screen navigation working |
| ScreenScan (spline editor) | Done | Catmull-Rom editor, playhead, dial — no real motion yet |
| ScreenLive | Done (mock) | UI complete, 10 Hz mock dynamics only |
| HTTP server :8080 | Done | REST API mirrors Qt state, web UI functional |
| Teensy firmware | Done | pendant_serial.py written, not yet wired to Qt |
| Pendant → UI bridge | Not started | pendant_serial.py needs threading into the Qt app |
| ClearCore TCP client | Not started | No TCP layer exists — largest outstanding gap |
| Real motor telemetry | Not started | MotorModel.tick() must be replaced with real data |
| ScreenCamera | Not started | New screen — requires camera API details first |
| ScreenPresets (full) | Stub only | Placeholder exists, no data model or logic |
| ScreenTelemetry (full) | Stub only | Placeholder exists, no graphs implemented |
| ScreenSettings (updated) | Needs update | Stale labels; persistence not implemented |
| Project folder structure | Done | Reorganised: pi/hmi, firmware, docs, electronics |
| Architecture document (PDF) | Done | docs/architecture/xylosome_architecture.pdf |